- coolant level fault code BOTH - I know we talked about this yesterday, but I did not realise at that time that the second machine had the same fault. I am meeting Caterpillar on site on Tuesday afternoon ( I am going to get them to force the regen through their Cat ET and make sure our signal for the enable is working as it should ) I will get them to see if the coolant level fault is a logged fault, or why their ECU is spitting out that message over CAN.

- DONE Manual Force when Tech logged in BOTH - Can we NOT ALLOW a forced output on the following OUT0200, OUT0201, OUT0203, OUT0204, OUT0205, OUT0104 - these are all for tracking outputs, I just feel that these would be better disabled for safety reasons if the Tech logged in had the engine started

- DONE Tech logged in BOTH - I would like the software changed so when a Tech is logged in then the siren/beacon output fires the whole time when logged in ( I know this will be somewhat a nuisance for the operator but I believe it just adds that higher level of warning/safety. Also a banner which flashes on the display saying something like "Maintenance Mode" or if you can think of something better.

- TBC Side conveyor raise/lower T620X - These are activating too early ( as soon as the feeder stop ) can we only fire these outputs when the machine stops all functions and lowers engine rpm back to idle again.

- DONE Jack legs raise/lower BOTH - These are activating too early ( as soon as the feeder stop ) can we only fire these outputs when the machine stops all functions and lowers engine rpm back to idle again.

- DONE - Auto tracking time delay BOTH - Can we extend the tracking output functions slightly. When in auto run mode and we press the tracking on/off button, for a bout 2-3 seconds before the tail conveyor fully stops I am to track.

- DONE Tracking mode rpm setting BOTH - when Tech logged in, and changing this setting, the max I can go is set at 1300rpm. It needs to be 1400.

- DONE E-stop On radio remote T620R - When in auto radial mode, we need the estop button to be active so the operator can shutdown machine when auto radial movement.

- DONE Manual slew left/right T620R - Can we add a bit to the software so if the machine is running in auto or manual ( NOT AUTO RADIAL -BELT JUST STAYING IN ONE POSITION ) if the operator presses either left or right input from radio then if the conveyor triggers either of the left or right limit switches then the movement stops,

- DONE also when running in auto mode and they press the left or right movement then can we change the software so that the machine does not go into auto stop mode.

- DONE Auto radial Mode T620R - if machine running in auto radial mode, then ignore inputs from left and right buttons on the radio remote to manually move conveyor either left or right.

Re: T620X/T620R Snags

A

[Draft]

This message hasn't been sent.

Saved: Tue 15/04/2025 17:11



Something went wrong and we haven't sent your message yet.

Need an engine idle rpm value - at 900 for now, plus do we allow jacklegs to work if swivel enabled/active?

Machine_IO.Inputs.SideConvRaise_RadialMoveLeft.Dig needs to be Machine_IO.Inputs.SideConvRaise_RadialMoveRight.Dig and so on..